Hercules Graphics

3B0h  synonym for 3B4h

3B1h  synonym for 3B5h

3B2h  synonym for 3B4h

3B3h  synonym for 3B5h

3B8h (W)

bit 1  Hercules graphics mode if set, text mode else

3  Video enabled if set

5  Blink enabled if set

7  Graphics page 1 displayed if set, page 0 else

3BAh (R)  Status Register

bit 0  Horizontal sync

3  Video signal

7  Vertical sync

3BAh (W)  Mode Select Register

bit 3  132-column text for monochrome

3BBh (W)  Lightpen Strobe Reset

Writing to this register will clear the lightpen strobe.

3BFh (W)

bit 0  Setting of graphics mode bit (3B8h bit 1) enabled if set

1  Setting of graphics page bit (3B8h bit 7) enabled if set

Hercules GRAFIX BIOS Interface:

----------1040-------------------------------

INT 10 - VIDEO - SET GRAPHICS MODE (Hercules GRAFIX)

AH = 40h

SeeAlso: AH=41h

----------1041-------------------------------

INT 10 - VIDEO - SET TEXT MODE (Hercules GRAFIX)

AH = 41h

SeeAlso: AH=40h

----------1042-------------------------------

INT 10 - VIDEO - CLEAR CURRENT PAGE (Hercules GRAFIX)

AH = 42h

SeeAlso: AH=45h

----------1043-------------------------------

INT 10 - VIDEO - SELECT DRAWING PAGE (Hercules GRAFIX)

AH = 43h

AL = page number (0,1)

SeeAlso: AH=44h,AH=45h

----------1044-------------------------------

INT 10 - VIDEO - SELECT DRAWING FUNCTION (Hercules GRAFIX)

AH = 44h

AL = drawing function

00h clear pixels

01h set pixels

02h invert pixels

SeeAlso: AH=44h,AH=46h,AH=4Ch,AH=4Dh

----------1045-------------------------------

INT 10 - VIDEO - SELECT PAGE TO DISPLAY (Hercules GRAFIX)

AH = 45h

AL = page number (0,1)

SeeAlso: AH=42h,AH=43h

----------1046-------------------------------

INT 10 - VIDEO - DRAW ONE PIXEL (Hercules GRAFIX)

AH = 46h

DI = x (0-720)

BP = y (0-347)

Note: function 44h determines operation and function 43h which page to use

SeeAlso: AH=0Ch,AH=47h,AH=49h,AH=4Ch,AH=4Dh

----------1047-------------------------------

INT 10 - VIDEO - FIND PIXEL VALUE (Hercules GRAFIX)

AH = 47h

DI = x (0-720)

BP = y (0-347)

Return: AL = 00h pixel clear

AL = 01h pixel set

Note: function 43h specifies which page is used

SeeAlso: AH=0Dh,AH=46h

----------1048-------------------------------

INT 10 - VIDEO - MOVE TO POINT (Hercules GRAFIX)

AH = 48h

DI = x (0-720)

BP = y (0-347)

SeeAlso: AH=49h

----------1049-------------------------------

INT 10 - VIDEO - DRAW TO POINT (Hercules GRAFIX)

AH = 49h

DI = x (0-720)

BP = y (0-347)

Note: function 48h or 49h specify first point, 44h operation and 43h page to

use

SeeAlso: AH=43h,AH=44h,AH=48h,AH=4Ch,AH=4Dh

----------104A-------------------------------

INT 10 - VIDEO - BLOCK FILL (Hercules GRAFIX)

AH = 4Ah

DI = x coordinate of lower left corner

BP = y coordinate of lower left corner

BX = height in pixels

CX = width in pixels

Note: draws a solid rectangle

SeeAlso: AH=4Eh

----------104B-------------------------------

INT 10 - VIDEO - DISPLAY CHARACTER (Hercules GRAFIX)

AH = 4Bh

AL = character to display

DI = x (0-720)

BP = y (0-347)

Note: unlike the other BIOS character functions character position is

specified in pixels rather than rows and columns

SeeAlso: AH=09h,AH=0Ah

----------104C-------------------------------

INT 10 - VIDEO - DRAW ARC (Hercules GRAFIX)

AH = 4Ch

AL = quadrant (1 = upper right, 2 = upper left, etc)

DI = x coordinate of center

BP = y coordinate of center

BX = radius

SeeAlso: AH=49h,AH=4Dh

----------104D-------------------------------

INT 10 - VIDEO - DRAW CIRCLE (Hercules GRAFIX)

AH = 4Dh

DI = x of center

BP = y of center

BX = radius

SeeAlso: AH=49h,AH=4Ch

----------104E-------------------------------

INT 10 - VIDEO - FILL AREA (Hercules GRAFIX)

AH = 4Eh

DI = x coordinate of an interior point

BP = y coordinate of an interior point

Notes: fills convex polygonal areas

the first fill makes the figure solid, the second erases it

SeeAlso: AH=4Ah